JORHAT INSTITUTE OF SCIENCE & TECHNOLOGY

B.Tech — Electronics & Telecommunication Engineering

SMART INDOOR AIR QUALITY MONITORING

An Embedded IoT System with Machine-Learning-Based Air Quality Classification

Project Report

MQ-135 • DHT11 • LDR Module  —  ESP8266 NodeMCU  →  Blynk IoT  →  ML Classifiers

Prepared by

Rishav Dutta

# 1. Abstract

This project implements a low-cost, end-to-end indoor air-quality monitoring system that combines embedded sensing, real-time IoT visualization, and offline machine learning. An ESP8266 NodeMCU samples an MQ-135 gas sensor, a DHT11 temperature/humidity sensor, and an LDR light module every 10 seconds, classifies air quality on-device, and streams the readings to a Blynk dashboard while logging them to Google Sheets. The logged data (1,499 readings) was exported as a CSV, cleaned in Google Colab, and used to train four classifiers — Logistic Regression, an Artificial Neural Network (ANN), Random Forest, and K-Nearest Neighbors (KNN) — to predict the air-quality label (Moderate / Poor). All four models achieved perfect scores (accuracy, precision, and F1 = 1.0) on a held-out test set, reflecting strong class separability in the current dataset, though the report also discusses the limitations behind that result and the steps recommended before real-world deployment.

# 2. Introduction

Indoor air quality directly affects health, comfort, and productivity, but continuous monitoring is often limited to expensive commercial sensors. This project explores whether a low-cost sensor stack — an MQ-135 gas sensor, a DHT11 temperature/humidity module, and a simple LDR light sensor — can be combined with an IoT dashboard and machine learning to deliver a usable, low-cost air-quality monitoring and classification system.

The system was designed and built as a B.Tech project at Jorhat Institute of Science & Technology, in the Electronics & Telecommunication Engineering department. The work spans three layers: embedded firmware and on-device classification, cloud-based live monitoring, and a machine-learning pipeline trained on the logged sensor history.

# 3. System Overview

The system follows a three-stage pipeline:

- Embedded Sensing — an ESP8266 NodeMCU reads the MQ-135 gas sensor, DHT11 temperature/humidity sensor, and LDR module every 10 seconds, then classifies air quality and flags gas anomalies directly on-device.

- IoT Dashboard + Logging — readings are streamed live to the Blynk app for remote monitoring, and simultaneously appended to a Google Sheet for long-term storage; that sheet is exported as the project's CSV dataset.

- Machine Learning — the logged CSV is loaded into Google Colab, cleaned and scaled, and used to train four classifiers (Logistic Regression, ANN, Random Forest, KNN) to predict the air-quality label.

# 4. Hardware and Embedded Implementation

## 4.1 Sensor Hardware

| Component | Interface | Function |
| --- | --- | --- |
| MQ-135 Gas Sensor | Analog — A0 | Raw ADC gas-concentration reading (0–1023); primary driver of the Air Quality classification |
| DHT11 Sensor | Digital — D4 | Ambient temperature (°C) and relative humidity (%) |
| LDR Module | Digital — D5 | Light state, used as a proxy for room occupancy |
| ESP8266 NodeMCU | Wi-Fi MCU | Samples every 10 s; pushes data to Blynk and Google Sheets |

Table 1. Sensor hardware used in the embedded node.

## 4.2 On-Device Classification Logic

Air quality is classified on-device directly from the MQ-135 raw ADC reading, using the following thresholds:

- Below 300 → Good

- 300–599 → Moderate

- 600 and above → Poor

This same label ('Air quality') is the target used later for training the machine-learning models on the logged sensor history. In addition, a secondary safety flag — Gas Anomaly — is raised whenever the MQ-135 raw value exceeds 600, independent of the air-quality classification.

# 5. Real-Time Dashboard (Blynk IoT)

Every 10 seconds, the NodeMCU writes seven parameters to Blynk virtual pins for live, remote monitoring:

- Gas ADC (MQ-135) — raw analog gas reading

- Temperature — from the DHT11, in °C

- Humidity — from the DHT11, shown on a radial gauge

- Light State — digital LDR output (0/1)

- Occupancy — estimated from the light state

- Air Quality Class — Good / Moderate / Poor

- Gas Event — binary anomaly flag

# 6. Dataset

The dataset consists of 1,499 sensor readings logged between 19 and 20 July 2026, captured at roughly 10-second intervals and exported from the Google Sheet as a CSV file. Each row records a timestamp, temperature, humidity, MQ-135 raw value, light state, the derived air-quality label, a gas-anomaly flag, and an occupancy status.

## 6.1 Class Distribution

| Air Quality Label | Count | Share |
| --- | --- | --- |
| Poor | 1,351 | 90.1% |
| Moderate | 148 | 9.9% |

Table 2. Air-quality label distribution across all 1,499 logged readings. The dataset is strongly imbalanced toward 'Poor', consistent with readings captured mostly in one elevated-MQ-135 environment; no 'Good' readings appear in this logged set.

The Gas Anomaly flag (MQ-135 > 600) is set on 1,351 of the 1,499 readings, closely tracking the Poor air-quality label since both are driven by the same underlying MQ-135 threshold logic. Occupancy Status was likewise skewed, with 1,349 readings logged as 'Unoccupied' and 150 as 'PossiblyOccupied'.

## 6.2 Sensor Value Ranges

| Feature | Min | Mean | Max |
| --- | --- | --- | --- |
| Temperature (°C) | 0.0* | 30.1 | 31.8 |
| Humidity (%) | 80 | 87.9 | 97 |
| MQ-135 raw value | 314 | 898.4 | 997 |

Table 3. Summary statistics for the numeric sensor features. *A small number of Temperature readings of 0.0 appear to be DHT11 read errors rather than true ambient values, and should be filtered in any future cleaning pass.

# 7. Data Preprocessing Pipeline

Before training, the logged CSV was processed in Colab through the following steps:

- Loaded the CSV of logged sensor readings into a pandas DataFrame.

- Parsed the Timestamp column into separate Date and Time features, each label-encoded.

- Dropped non-predictive or redundant columns (e.g., Occupancy Status).

- Label-encoded the target variable, Air quality (Moderate = 0, Poor = 1).

- Standardized the numeric features using scikit-learn's StandardScaler.

- Performed a stratified 80/20 train–test split on the Air quality target.

The resulting model used six input features: Temperature, Humidity, MQ-135 raw value, Light State, and the encoded Date and Time.

# 8. Model Training

All four classifiers were trained on the same six-feature, scaled dataset and the same stratified 80/20 split, giving 1,199 training samples and 300 test samples.

| Model | Configuration |
| --- | --- |
| Logistic Regression | scikit-learn LogisticRegression, max_iter = 1000 — a linear baseline classifier on the scaled features. |
| Artificial Neural Network | Keras Sequential: Dense(64, ReLU) → Dense(32, ReLU) → Dense(2, Softmax); Adam optimizer, sparse categorical cross-entropy, 50 epochs, batch size 32, 10% validation split. |
| Random Forest | scikit-learn RandomForestClassifier with default ensemble settings — captures non-linear feature interactions. |
| K-Nearest Neighbors | scikit-learn KNeighborsClassifier, default k = 5 — distance-based classification on standardized features. |

Table 4. Model configurations used for the four classifiers.

The ANN's training curves (accuracy and loss over 50 epochs) converged rapidly: both training and validation accuracy reached 1.0 within the first few epochs, and loss fell to near zero, indicating fast, stable learning and strong class separability on this dataset.

# 9. Evaluation and Results

All four models were evaluated on the same held-out 300-sample test set (30 Moderate, 270 Poor). Every model produced an identical confusion matrix, correctly classifying all 30 Moderate and all 270 Poor test samples with zero misclassifications.

| Model | Accuracy | Precision | F1 Score | Error |
| --- | --- | --- | --- | --- |
| Logistic Regression | 1.0 | 1.0 | 1.0 | 0.0 |
| Artificial Neural Network | 1.0 | 1.0 | 1.0 | 0.0 |
| Random Forest | 1.0 | 1.0 | 1.0 | 0.0 |
| K-Nearest Neighbors | 1.0 | 1.0 | 1.0 | 0.0 |

Table 5. Accuracy, precision (weighted), and F1 score (weighted) for all four classifiers on the 300-sample test set.

## 9.1 Reading the Results

The perfect scores across all four models should be interpreted with caution rather than as evidence of a solved problem. They reflect a small, controlled test setup and a strongly imbalanced dataset (about 90% Poor readings, captured mostly in one elevated-MQ-135 environment). This means the models likely learned an easy decision boundary rather than a robust one. The results demonstrate clean class separability on this particular dataset, but broader field testing across more environments and a more balanced dataset are recommended before drawing conclusions about real-world accuracy or attempting deployment.

# 10. Conclusion and Future Work

## 10.1 What Was Achieved

- An end-to-end pipeline was built: embedded sensing → live Blynk dashboard → CSV logging → multi-model training.

- A low-cost sensor stack (MQ-135, DHT11, LDR) reliably distinguishes Good / Moderate / Poor air quality using a simple threshold rule.

- Four models — Logistic Regression, ANN, Random Forest, and KNN — each achieved 1.0 accuracy, precision, and F1 on the Air Quality classification task.

## 10.2 Where This Goes Next

- Collect data across more environments to reduce class imbalance and add 'Good' air-quality examples.

- Calibrate the MQ-135 sensor to real PPM values instead of raw ADC thresholds.

- Cross-validate the models on a larger, more balanced dataset to stress-test the current perfect scores.

- Deploy the best-performing model on-device (TinyML) for real-time inference without a cloud round-trip.

# 11. Author

Rishav Dutta

Jorhat Institute of Science & Technology — B.Tech, Electronics & Telecommunication Engineering.